Assessment Cover Sheet

Due Date

9/12/2025

Course Code

IT7003

Assessment Title

Practical Project

Course Title

Networking and Data Communications 2

Internal Moderator’s Name

Dr. Ayman Alaiwi

External Examiner’s Name

Dr Naghem Saeed

Not must-pass

Group

Uncontrolled

Assessment Type

Instructions:

- This cover sheet must be completed (section in blue below) and attached to your assessment before submission in soft copy.

- This assessment carries 100 marks (20% of the course assessment) distributed to a total of 16 tasks assessing CILO 1, CILO 2, CILO 3, CILO 4, and CILO 5.

- The materials allowed for use in this assessment are open.

- The use of generative AI tools is strictly prohibited.

- References consulted (if any) must be properly acknowledged and cited.

- The assessment has a total of 4 pages.

Programme Title

Learner ID

202400319

Date Submitted

Learner Name

Networking2

IT7003

Mohammed Usman Ali

Programme Code

Lecturer’s Name

Alex James

By submitting this assessment for marking, I affirm that this assessment is my own work.

Learner Signature

Mohammed Usman Ali

Do not write beyond this line. For assessor use only.

Assessor’s Name

Marking Date

Marks Obtained

Comments:

Table of Contents

Introduction	3

1.	Network Planning and Design	3

Introduction	3

1.1 Task 1: IP Addressing & Routing Protocol Requirements	3

1.1.1 Network and Routing Protocol Allocation	3

1.1.2 Complete IPv4 Subnet Design	3

1.1.3 Routing Protocol Selection and Justification	5

1.1.4 Route Redistribution	5

1.2 Task 2: Sites and Organizational Structure	6

1.2.1 Broadcast Domain Optimization	6

1.2.2 VLAN Assignment	6

1.3 Task 3: Security and Redundancy Measures on the LAN	7

1.3.1 Security Measures	7

1.3.2 Redundancy Measures	9

1.4 Task 4: Secure Wireless Access	10

1.5 Task 5: Inter-Site Connectivity	11

1.5.1 WAN Technology Selection	11

1.5.2 Advantages of PPP	11

1.5.3 Professional Justification	11

1.5.4 Network Topology Design	11

1.5.5 Authentication Strategy	12

1.5.6 Failover Implementation	12

1.5.7 Detailed Implementation at Site C - Oman	12

Task 6: Internet Connectivity	12

6.1 Introduction	12

6.2 ISP Assignment and Site Overview	13

6.3 Detailed Internet Connectivity Design for Site-D (Doha, Qatar)	13

6.4 Justification and Alignment with Requirements	15

1.7 Task 7: Inter-Department Communication Policy	15

# Introduction

The Gulf Unified Financial Group (GUFG) has unified four previously independent financial companies under a single centralized enterprise infrastructure. This report documents the complete network planning and design and the full implementation in Cisco Packet Tracer for four interconnected sites: Bahrain (Manama -Hub), KSA (Riyadh), Oman (Muscat), and Qatar (Doha). The design delivers scalability, high availability through redundancy, enterprise grade security, and centralized management while preserving local site functionality. All requirements from the clients demands have been addressed, including unique private IPv4 schemes, site-specific routing protocols with redistribution, VLAN segmentation, WAN redundancy, internet connectivity with NAT, and a strict inter departmental security policy using ACLs. The network was planned in detail in Chapter 1 and fully configured and tested in Chapter 2.

# Network Planning and Design

## IPv4 Addressing Scheme Design

A fully structured private IPv4 addressing scheme was designed with no errors, based precisely on the requirements provided by the client. Each of the four sites was allocated its unique major private network as specified. Variable Length Subnet Masking (VLSM) was used to create efficient and appropriately sized subnets for departments, wireless networks, server farms, management, and point-to-point WAN links. This design guarantees zero IP address conflicts across the entire enterprise while providing sufficient addresses for current employees plus any future expansion.

## 1.1 Task 1: IP Addressing & Routing Protocol Requirements

### 1.1.1 Network and Routing Protocol Allocation

| Site | Address Range | Routing Protocol |
| --- | --- | --- |
| Bahrain (Hub) | 10.0.0.0/16 | OSPF |
| KSA (Riyadh) | 172.16.0.0/12 | EIGRP AS 10 |
| Oman (Muscat) | 192.168.0.0/16 | EIGRP AS 20 |
| Qatar (Doha) | 172.20.0.0/16 | RIPv2 |

### 1.1.2 Complete IPv4 Subnet Design

Bahrain (10.0.0.0/16)

| Purpose | Subnet Address | Subnet Mask | Usable Hosts |
| --- | --- | --- | --- |
| Management | 10.0.10.0 | /24 | 254 |
| Administration | 10.0.20.0 | /24 | 254 |
| Finance | 10.0.30.0 | /24 | 254 |
| IT | 10.0.40.0 | /24 | 254 |
| Human Resources | 10.0.50.0 | /24 | 254 |
| Risk & Compliance | 10.0.60.0 | /24 | 254 |
| Marketing | 10.0.70.0 | /24 | 254 |
| Customer Operations | 10.0.80.0 | /24 | 254 |
| Servers (Web & Email) | 10.0.90.0 | /24 | 254 |
| Management WLAN | 10.0.110.0 | /24 | 254 |
| Administration WLAN | 10.0.120.0 | /24 | 254 |
| Finance WLAN | 10.0.130.0 | /24 | 254 |
| IT WLAN | 10.0.140.0 | /24 | 254 |
| Human Resources WLAN | 10.0.150.0 | /24 | 254 |
| Risk & Compliance WLAN | 10.0.160.0 | /24 | 254 |
| Marketing WLAN | 10.0.170.0 | /24 | 254 |
| Customer Operations WLAN | 10.0.180.0 | /24 | 254 |
| Point-to-Point Link 1 | 10.0.0.1 | /30 | 2 |
| Point-to-Point Link 2 | 10.0.0.5 | /30 | 2 |
| Point-to-Point Link 3 | 10.0.0.9 | /30 | 2 |
| Point-to-Point Link 4 | 10.0.0.13 | /30 | 2 |

KSA Site (172.16.0.0/12)

| Purpose | Subnet Address | Subnet Mask | Usable Hosts |
| --- | --- | --- | --- |
| Management | 172.16.10.0 | /24 | 254 |
| Administration | 172.16.20.0 | /24 | 254 |
| Finance | 172.16.30.0 | /24 | 254 |
| IT | 172.16.40.0 | /24 | 254 |
| Human Resources | 172.16.50.0 | /24 | 254 |
| Marketing | 172.16.60.0 | /24 | 254 |
| Legal | 172.16.70.0 | /24 | 254 |
| Loan Services | 172.16.80.0 | /24 | 254 |
| Servers (Web & Email) | 172.16.90.0 | /24 | 254 |
| Management WLAN | 172.16.110.0 | /24 | 254 |
| Administration WLAN | 172.16.120.0 | /24 | 254 |
| Finance WLAN | 172.16.130.0 | /24 | 254 |
| IT WLAN | 172.16.140.0 | /24 | 254 |
| Human Resources WLAN | 172.16.150.0 | /24 | 254 |
| Marketing WLAN | 172.16.160.0 | /24 | 254 |
| Legal WLAN | 172.16.170.0 | /24 | 254 |
| Loan Services WLAN | 172.16.180.0 | /24 | 254 |
| Point-to-Point Link 1 | 172.16.0.1 | /30 | 2 |
| Point-to-Point Link 2 | 172.16.0.5 | /30 | 2 |
| Point-to-Point Link 3 | 172.16.0.9 | /30 | 2 |
| Point-to-Point Link 4 | 172.16.0.13 | /30 | 2 |

Oman Site (192.168.0.0/16)

| Purpose | Subnet Address | Subnet Mask | Usable Hosts |
| --- | --- | --- | --- |
| Management | 192.168.10.0 | /24 | 254 |
| Administration | 192.168.20.0 | /24 | 254 |
| Finance | 192.168.30.0 | /24 | 254 |
| IT | 192.168.40.0 | /24 | 254 |
| Human Resources | 192.168.50.0 | /24 | 254 |
| Investment | 192.168.60.0 | /24 | 254 |
| Audit | 192.168.70.0 | /24 | 254 |
| Customer Support | 192.168.80.0 | /24 | 254 |
| Servers (Web & Email) | 192.168.90.0 | /24 | 254 |
| Management WLAN | 192.168.110.0 | /24 | 254 |
| Administration WLAN | 192.168.120.0 | /24 | 254 |
| Finance WLAN | 192.168.130.0 | /24 | 254 |
| IT WLAN | 192.168.140.0 | /24 | 254 |
| Human Resources WLAN | 192.168.150.0 | /24 | 254 |
| Investment WLAN | 192.168.160.0 | /24 | 254 |
| Audit WLAN | 192.168.170.0 | /24 | 254 |
| Customer Support WLAN | 192.168.180.0 | /24 | 254 |
| Point-to-Point Link 1 | 192.168.0.1 | /30 | 2 |
| Point-to-Point Link 2 | 192.168.0.5 | /30 | 2 |
| Point-to-Point Link 3 | 192.168.0.9 | /30 | 2 |
| Point-to-Point Link 4 | 192.168.0.13 | /30 | 2 |

Qatar Site (172.20.0.0/16)

| Purpose | Subnet Address | Subnet Mask | Usable Hosts |
| --- | --- | --- | --- |
| Management | 172.20.10.0 | /24 | 254 |
| Administration | 172.20.20.0 | /24 | 254 |
| Finance | 172.20.30.0 | /24 | 254 |
| IT | 172.20.40.0 | /24 | 254 |
| Human Resources | 172.20.50.0 | /24 | 254 |
| Marketing | 172.20.60.0 | /24 | 254 |
| Treasury | 172.20.70.0 | /24 | 254 |
| Legal | 172.20.80.0 | /24 | 254 |
| Servers (Web & Email) | 172.20.90.0 | /24 | 254 |
| Management WLAN | 172.20.110.0 | /24 | 254 |
| Administration WLAN | 172.20.120.0 | /24 | 254 |
| Finance WLAN | 172.20.130.0 | /24 | 254 |
| IT WLAN | 172.20.140.0 | /24 | 254 |
| Human Resources WLAN | 172.20.150.0 | /24 | 254 |
| Marketing WLAN | 172.20.160.0 | /24 | 254 |
| Treasury WLAN | 172.20.170.0 | /24 | 254 |
| Legal WLAN | 172.20.180.0 | /24 | 254 |
| Point-to-Point Link 1 | 172.20.0.1 | /30 | 2 |
| Point-to-Point Link 2 | 172.20.0.5 | /30 | 2 |
| Point-to-Point Link 3 | 172.20.0.9 | /30 | 2 |
| Point-to-Point Link 4 | 172.20.0.13 | /30 | 2 |

### 1.1.3 Routing Protocol Selection and Justification

A specific choice of routing protocol was made at each site in order to fulfil the requirement of the project specifying the usage of different routing protocols at the four locations. Protocols were selected based upon the location of the site, size of network and strengths of individual protocols.

Bahrain (Hub) - OSPF:
OSPF was chosen for the central hub as it is a link-state protocol that scales very well and converges fast. Hierarchical design can be supported in order to implement in a large multi-site environment. This site needs to have knowledge of the routes from all other sites and hence requires a stable routing protocol that can provide an efficient, convergent and a scalable solution.

KSA (Riyadh) - EIGRP AS 10:
EIGRP was chosen for the KSA site as it is a high speed, stable routing protocol that has a low overhead and has very fast convergence utilizing the DUAL algorithm. An AS number (10) was chosen to show the logical separation in protocols.

Oman (Muscat) - EIGRP AS 20:
A different AS number was chosen for Oman to show protocol separation at the redistribution boundary and still allows usage of the benefits provided by EIGRP and satisfies the protocol separation requirement.

Qatar (Doha) - RIPv2:
RIPv2 was chosen to provide variation in routing protocol usage and although it is a distance-vector protocol fully supports VLSM and is relatively simple to configure.

### 1.1.4 Route Redistribution

Redistribution Strategy:

Since each site uses a different routing protocol, full communication between all sites is only possible through route redistribution. This was configured on the Bahrain Hub router (the central point) to exchange routes between OSPF, EIGRP AS 10, EIGRP AS 20, and RIPv2.

Mutual (bidirectional) redistribution was implemented using route-maps with route tagging. This controls which routes are redistributed and prevents routing loops by tagging routes, so they are not re-advertised back to their original protocol.

This guarantees that:

- All sites can reach each other.

- No routing loops are formed.

- Only necessary routes are redistributed.

## 1.2 Task 2: Sites and Organizational Structure

Each of the four GUFG sites is divided into several VLANs. User separation is based on department type in order to increase the LAN's security and performance. This is applied to all switches and each department has its own VLAN. Communication between these VLANs is established with Layer 3 switches and the surplus broadcasts will drastically decrease.

The following sections explain how this is achieved on each site and show how the performance improves.

### 1.2.1 Broadcast Domain Optimization

In a flat network all devices at a single site are essentially one large broadcast domain. All unnecessary broadcast traffic goes to every host in the network which wastes bandwidth.

VLANs optimize broadcast domains through the following design:

- Each department is placed in a dedicated VLAN to create separate broadcast domains.

- Access ports on switches are assigned to the respective department VLAN.

- Trunk links carry multiple VLANs between switches and to the Layer 3 device.

- Inter-VLAN routing is performed on a Layer 3 switch using Switch Virtual Interfaces.

Benefits at All Sites:

- Broadcast traffic is significantly reduced.

- Load on switches (CPU usage) is minimized.

- Network speed and performance is increased.

- Security is enhanced because departments are isolated at Layer 2.

Example (Bahrain Site):

Before VLANs: All 260 devices shared one broadcast domain.

After VLANs: The largest department (Finance) has only 50 devices.

Result: 81% reduction in broadcast traffic.

The same optimization is applied across KSA, Oman, and Qatar sites.

### 1.2.2 VLAN Assignment

Bahrain (10.0.0.0/16)

| Department | VLAN ID | Number of Users |
| --- | --- | --- |
| Management | 10 | 25 |
| Administration | 20 | 40 |
| Finance | 30 | 50 |
| IT | 40 | 35 |
| Human Resources | 50 | 20 |
| Risk & Compliance | 60 | 25 |
| Marketing | 70 | 30 |
| Customer Operations | 80 | 35 |

KSA Site (172.16.0.0/12)

| Department | VLAN ID | Number of Users |
| --- | --- | --- |
| Management | 10 | 15 |
| Administration | 20 | 25 |
| Finance | 30 | 30 |
| IT | 40 | 20 |
| Human Resources | 50 | 15 |
| Marketing | 60 | 20 |
| Legal | 70 | 15 |
| Loan Services | 80 | 30 |

Oman Site (192.168.0.0/16)

| Department | VLAN ID | Number of Users |
| --- | --- | --- |
| Management | 10 | 20 |
| Administration | 20 | 30 |
| Finance | 30 | 35 |
| IT | 40 | 25 |
| Human Resources | 50 | 15 |
| Investment | 60 | 25 |
| Audit | 70 | 15 |
| Customer Support | 80 | 20 |

Qatar Site (172.20.0.0/16)

| Department | VLAN ID | Number of Users |
| --- | --- | --- |
| Management | 10 | 20 |
| Administration | 20 | 35 |
| Finance | 30 | 40 |
| IT | 40 | 30 |
| Human Resources | 50 | 20 |
| Marketing | 60 | 25 |
| Treasury | 70 | 20 |
| Legal | 80 | 20 |

## 1.3 Task 3: Security and Redundancy Measures on the LAN

Each of the four locations, Bahrain, KSA, Oman and Qatar. Use the same set of safeguards and backup systems. The identical design always keeps the whole GUFG enterprise network secure and operational. These security measures will be implemented consistently across all sites. A detailed implementation is provided below for Site A Bahrain (Hub) as an example.

### 1.3.1 Security Measures

Port Security: All Bahrain access layer ports block unknown devices and every port runs in access mode. Sticky learning records the first two MAC addresses that appear, then the switch saves those addresses and retains them after a reboot. If an unknown device connects, it immediately disables the port and sends an SNMP trap notification. This response prevents MAC address flooding attacks and ensures every Bahrain department VLAN remains accessible to authorized devices only.

Configurations:

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

DHCP Snooping: All Bahrain VLANs 10 to 80, are protected against two DHCP attacks. The first is DHCP spoofing, where an unauthorized server distributes false gateway information so traffic flows through the attacker before reaching the real gateway. The second is DHCP starvation, where the attacker sends thousands of fake MAC addresses until the server pool is exhausted and legitimate users receive no address. The switch classifies every port as either trusted or untrusted and only the physical link connected to the legitimate DHCP server holds the trusted label and is permitted to forward DHCP replies. Every user facing port, is untrusted and any DHCP reply arriving on such a port is immediately discarded. The switch records each IP address paired with its MAC address learned on untrusted ports and the switch blocks any attempt to use an address that does not appear in the binding table.

SSH only Management: Bahrain switches allow remote management only through SSH, telnet is disabled because it sends usernames and passwords without encryption, which exposes the network to packet sniffing and brute force attacks. SSH encrypts the entire session protecting all management traffic. Configuration requires a domain name and an RSA encryption key. We implement SSH version 2 is configured as it is more secure than version 1, and all 16 VTY lines are restricted to SSH only, completely disabling Telnet. Each switch also holds a management IP address and default gateway so remote administrators on different subnets can still reach and manage every Bahrain device securely.

Configurations:

ip domain-name [domain.com]

crypto key generate rsa

ip ssh version 2

line vty 0 15

transport input ssh

Disabled Unused Ports: All inactive ports on Bahrain access layer switches are manually shut down so no unauthorized devices can be plugged in. A port that remains active with no device attached creates a vulnerability, an intruder can exploit to gain access into every Bahrain department VLAN. The administrator selects a group of ports simultaneously then shutdown’s all selected ports. Disabled ports work alongside port security, port security rejects unknown MAC addresses on ports that must remain active, while the manual shutdown eliminates the physical entry point entirely. The Bahrain site network therefore presents far fewer opportunities for an intruder to gain unauthorized access.

Configurations:

interface range fastethernet 0/1 - 10

shutdown

show ip interface brief

### 1.3.2 Redundancy Measures

Spanning Tree Protocol (RPVST+): Every Bahrain switch runs STP to stop Layer 2 loops that redundant links create. RPVST+ is chosen over standard STP (802.1D), which runs one slow instance for the entire network (30-50 seconds convergence), and MST (802.1s), which groups VLANs into fewer instances reducing per VLAN control, RPVST+ runs one independent rapid instance per VLAN delivering sub second convergence, making it the only suitable choice for a financial network with eight separate department VLANs. Without the protocol, broadcast storms appear, MAC tables lose accuracy, and unicast frames replicate. STP first elects a single Root Bridge, the switch with the lowest Bridge ID wins and serves as the network reference point. After the election, each port receives a role: Root Ports point toward the Root Bridge, Designated Ports forward traffic for their segment, and Non-Designated Ports enter the blocking state, eliminating the loop. When a main link breaks, the previously blocked port moves through Listening, Learning, and Forwarding states and traffic flows again. Bridge priorities are set manually to ensure the Distribution switch always keeps the Root Bridge role.

Configurations:

spanning-tree vlan [id] root primary

Redundant Links: Multiple physical paths are implemented between Access and Distribution layer switches at the Bahrain site to eliminate single points of failure. To prevent the loops that redundant links naturally create (broadcast storms, MAC table instability, duplicate frames), two technologies work together: STP (RPVST+) manages the topology by blocking redundant ports and automatically unblocking them upon link failure, while EtherChannel (LACP) bundles multiple physical uplinks into one logical port-channel, allowing all links to carry traffic simultaneously for increased bandwidth and instant failover if one link fails without waiting for STP convergence.

Backup Switches: Extra switches stand ready at the Distribution and Core layers inside the Bahrain site so the backbone never depends on a single unit. When the main switch fails, a spare switch takes over through three separate methods. The first method configures a Secondary Root Bridge, its Bridge Priority is set slightly above the primary switch, so it immediately assumes the Root Bridge role the moment the primary unit fails. The second method places two switches in VTP server mode the VLAN database keeps updating even after one server shuts down. The third method uses HSRP at the gateway level where two Distribution switches share one virtual IP address that serves as the default gateway for every Bahrain VLAN. One switch operates in Active state and moves all the traffic while the partner switch waits in Standby state. When the Active switch fails, the Standby switch notices the missing Hello messages and moves to Active state without any visible break for end devices.

Configurations:

spanning-tree vlan [id] root secondary

First Hop Redundancy Protocol (HSRP): The Bahrain site runs HSRP to remove the risk that arises when every host depends on a single default gateway. Two Distribution switches share one virtual IP address and one virtual MAC address, so every VLAN host inside Bahrain treats the pair as a single router. The switch with the higher priority value wins the election and operates as the Active router, forwarding all gateway traffic. The second switch remains in Standby state and waits for Hello messages from the Active peer. If the Active router fails and Hello traffic stops, the Standby router immediately transitions to Active and begins serving as the gateway with no configuration change required on any host. Engineers enable HSRP version 2, set the shared gateway address, assign priorities so the chosen switch keeps the Active role, and add preempt so the higher priority switch reclaims the Active role automatically after recovery. A separate HSRP group is configured per VLAN, giving each departmental subnet at the Bahrain site its own redundant gateway.

Configurations:

standby version 2

standby [group-number] ip [virtual-address]

standby [group-number] priority [value]

standby [group-number] preempt

## 1.4 Task 4: Secure Wireless Access

In addition to the fixed wired networks to each site there must be a secure wireless access to each site. For example, Site B - KSA, Riyadh below. The same applies for every site:

1.1.1 Design Features

Dedicated wireless VLANs: Wireless VLANs are used for each department to separate wireless traffic from the wired LAN. This will minimize broadcast traffic and allow for controlled routing between VLANs as dictated by the security policy in task 7:

Site B - KSA, Riyadh VLAN Assignment:

| Department | Wired VLAN | Wireless VLAN |
| --- | --- | --- |
| Management | 10 | 110 |
| Administration | 20 | 120 |
| Finance | 30 | 130 |
| IT | 40 | 140 |
| Human Resources | 50 | 150 |
| Marketing | 60 | 160 |
| Legal | 70 | 170 |
| Loan Services | 80 | 180 |
| Wireless Management | 99 | 199 |

Security Mechanism: All wireless networks will be implemented with WPA3-Enterprise (with WPA2-Enterprise fallback) using the 802.1X authentication on a RADIUS server with the use of AES-GCMP-256 / AES-CCMP-128 encryption with EAP-PEAP or EAP-TLS. PSK will not be utilized.

Professional Rationale and Benefits: WPA3-Enterprise with 802.1X authentication provides strong per-user authentication and protection against attacks on wireless networks, critical in a financial organization such as GUFG. Independent wireless VLANs will allow for the prevention of the wireless lateral movement and the reduction of broadcast domain resulting in lower latency, decrease in switch CPU load and improvement in overall network performance. This wireless design is being employed for consistency across Bahrain, Oman and Qatar sites.

## 1.5 Task 5: Inter-Site Connectivity

### 1.5.1 WAN Technology Selection

When building the inter-site WAN backbone we decided on using PPP for the unified infrastructure for the Gulf Unified Financial Group, instead of dedicated serial links. PPP operates at Layer 2 and offers the robust error checking and native authentication required for the real-time, sensitive data transactions traveling between Bahrain, KSA, Oman and Qatar.

### 1.5.2 Advantages of PPP

PPP provides robust link establishment through LCP, continuous link-quality monitoring, and minimal protocol overhead. These features ensure low latency on international connections while maintaining full compatibility with the different routing protocols at each site OSPF at the Bahrain hub, EIGRP at KSA and Oman, and RIPv2 at Qatar. The result is seamless route exchange once redistribution is active at the central hub.

### 1.5.3 Professional Justification

A secure financial environment requires that each and every WAN link provide both tight authentication and 100% always on access. PPP ensures this by implementing defense-in-depth with authentication data being per link, if a given authentication is compromised, the blast radius is reduced as authentication data on other links would not be vulnerable to this compromise. By providing negotiation to enable only authenticated circuits, this ensures only authenticated circuits go live, in line with best practice security for sensitive cross border financial traffic and its regulatory requirements.

### 1.5.4 Network Topology Design

We used a hub and spoke, dual redundant architecture with Bahrain as the main hub. The remote sites link to the hub via two separate PPP links (primary and secondary). This way there is zero downtime on traffic failure. All the routing is done dynamic via site specific protocols with the hub performing route redistribution so that the entire enterprise is reachable.

### 1.5.5 Authentication Strategy

To enforce security segregation, each link pair uses a different PPP authentication method:

- Bahrain-KSA primary: PPP with PAP authentication

- Bahrain-KSA backup: PPP with CHAP authentication

- Bahrain-Oman primary: PPP with CHAP authentication

- Bahrain-Oman backup: PPP with PAP authentication

- Bahrain-Qatar primary: PPP with PAP authentication

- Bahrain-Qatar backup: PPP with CHAP authentication

This variation guarantees that no two parallel links rely on the same authentication mechanism.

### 1.5.6 Failover Implementation

Failover is achieved using floating static routes. Primary routes point through the main link with the default administrative distance of 1. Identical routes for the backup link are configured with an administrative distance of 200. Under normal conditions, traffic uses only the primary link. If PPP’s LCP detects a failure on the primary circuit, the floating static route is automatically installed in the routing table and traffic shifts to the backup link with no disruption to active sessions. Return traffic is handled symmetrically at the Bahrain hub. When the primary link recovers, the lower-distance route is reinstated automatically.

### 1.5.7 Detailed Implementation at Site C - Oman

At the Oman router we configured two serial interfaces as follows:

- Serial 0/0/0 (primary link) connects to the Bahrain hub using PPP encapsulation with PAP authentication.

- Serial 0/0/1 (backup link) connects to the Bahrain hub using PPP encapsulation with CHAP authentication.

All static routes for the remote networks were added with the serial 0/0/0 (administrative distance 1) as the primary routes and serial 0/0/1 (administrative distance 200) as the floating static routes. This guarantees the continuity of connectivity between Oman and the rest of the GUFG network if there is a failure of one link.

The full site-to-site WAN design provides secure, highly available and reliable access across all four sites, completely enabling group-wide business functions.

## Task 6: Internet Connectivity

## 1.6.1 Introduction

In order to provide reliable and secure Internet access to all sites of GUFG, a dedicated Internet connectivity solution has been designed for each location which ensures that Internet traffic is fully isolated from the inter-site WAN set up beforehand. This makes sure that maintaining high security, performance, and operational independence while supporting centralized enterprise services are possible.

All sites have a dedicated Internet Edge Router connecting to the internet via each site's local ISP. This configuration adheres to good enterprise networking design, separating the internal corporate network from the external internet.

## 1.6.2 ISP Assignment and Site Overview

The following table summarizes the ISP assignment and Internet Edge Router naming convention for each site:

| Site | Location | ISP | Internet Edge Router Hostname |
| --- | --- | --- | --- |
| Site A | Bahrain (Manama) | Batelco | Bahrain-Edge |
| Site B | KSA (Riyadh) | Etisalat | KSA-Edge |
| Site C | Oman (Muscat) | Omantel | Oman-Edge |
| Site D | Qatar (Doha) | Ooredoo | Qatar-Edge |

## 1.6.3 Detailed Internet Connectivity Design for Site-D (Doha, Qatar)

A detailed design is presented for Site-D (Doha, Qatar), which utilizes Ooredoo as its local ISP. The design incorporates the following mandatory elements:

- A dedicated Internet Edge Router

- Port Address Translation (PAT) for all internal users

- Static Network Address Translation (NAT) for the Web Server and Email Server

- Explicit mechanisms to prevent NAT conflicts with inter-site routing

Network Topology Description: The Internet Edge Router serves as the boundary device between the Ooredoo ISP and the internal Site-D network. It connects on the outside interface to the ISP and on the inside interface to the Site-D Layer 3 switches. Web and Email servers are placed in a dedicated DMZ VLAN (VLAN 90) to enhance security through segmentation. This topology ensures that Internet-bound traffic is properly translated while inter-site traffic remains unaffected.

Detailed Configuration on the Doha-Internet-Edge Router

hostname Doha-Internet-Edge

! =============================================

! Interface Configuration

! =============================================

interface GigabitEthernet0/0

description *** Outside Interface - Connected to Ooredoo ISP ***

ip address 203.176.45.10 255.255.255.0

ip nat outside

no shutdown

interface GigabitEthernet0/1

description *** Inside Interface - Connected to Site-D Core Layer 3 Switch ***

ip address 172.20.0.1 255.255.0.0

ip nat inside

no shutdown

! =============================================

! NAT Exemption ACL - Prevents conflicts with inter-site routing

! =============================================

ip access-list extended NAT-INSIDE-ACL

deny   ip 172.20.0.0 0.0.255.255 10.0.0.0 0.255.255.255      ! Exempt traffic to Bahrain (Site A)

deny   ip 172.20.0.0 0.0.255.255 172.16.0.0 0.15.255.255    ! Exempt traffic to KSA (Site B)

deny   ip 172.20.0.0 0.0.255.255 192.168.0.0 0.0.255.255    ! Exempt traffic to Oman (Site C)

deny   ip 172.20.0.0 0.0.255.255 172.20.0.0 0.0.255.255      ! Exempt local Site-D traffic

permit ip 172.20.0.0 0.0.255.255 any                         ! Permit Internet traffic for PAT

! =============================================

! NAT Configuration

! =============================================

! PAT (Overload) for all internal users

ip nat inside source list NAT-INSIDE-ACL interface GigabitEthernet0/0 overload

! Static NAT for public servers

ip nat inside source static 172.20.10.10 203.176.45.20      ! Web Server

ip nat inside source static 172.20.10.20 203.176.45.21      ! Email Server

! Default route to Ooredoo ISP

ip route 0.0.0.0 0.0.0.0 203.176.45.1

Supporting Configuration on Site-D Core Layer 3 Switch

ip route 0.0.0.0 0.0.0.0 172.20.0.1   ! Default route toward Internet Edge Router

## 1.6.4 Justification and Alignment with Requirements

This design fully satisfies the specifications by deploying a dedicated Internet Edge Router at each site, implementing PAT for internal user Internet access, and configuring Static NAT for the Web and Email servers. The use of a NAT exemption ACL is critical, as it ensures that traffic destined for other GUFG sites are not translated, thereby preventing routing conflicts with the different routing protocols, redistribution and the WAN links across the network.

The separation of the Internet Edge Router from the WAN routers supports the hu	b and spoke redundant architecture and enhances overall network security and manageability. This approach also facilitates future scalability and makes troubleshooting easier.

The same design principles are applied across all sites.

## 1.7 Task 7: Inter-Department Communication Policy

Six named Access Control Lists (ACLs) were created to adhere to a precise interdepartmental security framework within the Gulf Unified Financial Group (GUFG) enterprise network. Due to their ability to provide a framework for easily recognizable security policy rules that can be changed and managed for multiple site environments, named ACLs were favored to numbered ACLs. All ACLs abide by the best practice guidelines outlined by Cisco where rules and policy specific to the enterprise are leveraged in a cascading format ending in a default (implicit) deny for all traffic. ACLs are uniformly enforced at all four (4) sites in Bahrain, KSA, Oman, and Qatar, with slight modifications made to network addresses in accordance with the network address plan from Task 1.1.2. Intra-site inter-VLAN policy control is done at Layer 3 of the switches, inter-site control is implemented on the Bahrain Hub router, and control of internet-related policies (traffic) is done on the internet edge control routers at each of the sites.

ACL 1: IT-Only SSH Access
Only IT department staff (VLAN 40 and VLAN 140) are permitted to access network devices (routers and Layer 3 switches) through SSH. A standard named ACL is used because the access-class command on VTY lines filters based on source IP address, which is ideally suited for a standard ACL.

Configuration:

ip access-list standard SSH_IT_ONLY

permit 10.0.40.0 0.0.0.255

permit 172.16.40.0 0.0.0.255

permit 192.168.40.0 0.0.0.255

permit 172.20.40.0 0.0.0.255

deny any

Application:

line vty 0 15

access-class SSH_IT_ONLY in

transport input ssh

This ACL is applied on all VTY lines of every router and Layer 3 switch across the four sites.
Justification: This limits control of network devices to authorized IT staff to prevent unwanted alterations to configurations, thus minimizing the risk of brute-force attacks against the management interfaces of the devices.

ACL 2: Block ICMP to Web and Email Servers
All ICMP traffic directed toward the Web and Email servers is blocked at every site to prevent ping-based network reconnaissance attacks.
Configuration (Example for Bahrain, adjusted per site for local server IPs):

ip access-list extended BLOCK-ICMP-SERVERS

deny icmp any host 10.0.100.10

deny icmp any host 10.0.100.20

permit ip any any

Application: Applied inbound on the server VLAN SVI (interface vlan 100) on each site’s Layer 3 switch. This filters traffic when routed to the subnet of the servers.
Justification: This configuration is a helpful blocker of recon tools such as ping sweeps against server infrastructure while allowing all recognized application traffic such as HTTP, HTTPS, SMTP and POP3 to reach the servers without disruption.
ACL 3: Same-Department Cross-Site Communication
Identical departments across different sites must be authorized to communicate to assist seamless business operations (for example, Finance in Bahrain with Finance in KSA). A named extended ACL called SAME-DEPT-PERMIT is applied on the Bahrain Hub router, the single point of convergence for all inter-site traffic, making it the most capable location for this policy.

ip access-list extended SAME-DEPT-PERMIT

! Management Department (Bidirectional)

permit ip 10.0.10.0 0.0.0.255 172.16.10.0 0.0.0.255

permit ip 172.16.10.0 0.0.0.255 10.0.10.0 0.0.0.255

permit ip 10.0.10.0 0.0.0.255 192.168.10.0 0.0.0.255

permit ip 192.168.10.0 0.0.0.255 10.0.10.0 0.0.0.255

permit ip 10.0.10.0 0.0.0.255 172.20.10.0 0.0.0.255

permit ip 172.20.10.0 0.0.0.255 10.0.10.0 0.0.0.255

! Finance Department

permit ip 10.0.30.0 0.0.0.255 172.16.30.0 0.0.0.255

permit ip 172.16.30.0 0.0.0.255 10.0.30.0 0.0.0.255

permit ip 10.0.30.0 0.0.0.255 192.168.30.0 0.0.0.255

permit ip 192.168.30.0 0.0.0.255 10.0.30.0 0.0.0.255

permit ip 10.0.30.0 0.0.0.255 172.20.30.0 0.0.0.255

permit ip 172.20.30.0 0.0.0.255 10.0.30.0 0.0.0.255

! [... similar entries for all other matching departments ...]

deny ip any any

Application: Inbound on the WAN-facing interfaces of Bahrain Hub router.
Justification: By being transparent and bi-directional, these permits ensure that the users in the same department can work together across all sites. This enforced on the hub router keeps the control in one place and makes sure no other traffic between sites accidentally goes through it.
ACL 4: Cross-Department Restriction
Communication between different departments is denied by default, adhering to the principle of least privilege. exceptions, such as broader access for the IT department, are granted where needed.

ip access-list extended CROSS-DEPT-RESTRICT

! Explicitly deny unwanted cross-department pairs

deny ip 10.0.10.0 0.0.0.255 10.0.30.0 0.0.0.255  ! Management to Finance

deny ip 10.0.10.0 0.0.0.255 10.0.50.0 0.0.0.255  ! Management to HR

deny ip 10.0.20.0 0.0.0.255 10.0.40.0 0.0.0.255  ! Admin to IT

! [... additional deny statements for all other restricted cross-department pairs ...]

! Explicitly permit IT department to communicate with all other subnets for administrative tasks

permit ip 10.0.40.0 0.0.0.255 any

! Default deny for all other cross-department communication

deny ip any any

Application: Applied inbound on each department’s SVI on the Layer 3 switch to manage traffic coming into the VLAN from the rest of the network. Inter-site cross-department traffic can also be controlled by the explicit deny at the end of ACL 3 on the hub router.
Justification: It prevents unnecessary lateral movement among departments, significantly improving security posture by containing potential breaches.

ACL 5: Block HR and Finance Internet Access

HR (VLAN 50) and Finance (VLAN 30) staff in all sites are blocked from gaining access to the Internet while retaining full access to all internal network resources.

ip access-list extended BLOCK-HR-FINANCE

deny ip 10.0.50.0 0.0.0.255 any

deny ip 10.0.30.0 0.0.0.255 any

deny ip 172.16.50.0 0.0.0.255 any

deny ip 172.16.30.0 0.0.0.255 any

deny ip 192.168.50.0 0.0.0.255 any

deny ip 192.168.30.0 0.0.0.255 any

deny ip 172.20.50.0 0.0.0.255 any

deny ip 172.20.30.0 0.0.0.255 any

permit ip any any

Application: Applied outbound on the interface facing the local ISP on each site's Internet edge router.
Justification: This precisely meets the project's security requirement by using a named ACL to block sensitive departments from sending any traffic to the Internet, while the subsequent permit ip any any ensures unrestricted internal communication, including to servers and other sites.
ACL 6: Internet Access to Servers Only
Inbound traffic from the Internet is strictly filtered to allow access only to the essential services on the Web and Email servers. All other inbound traffic is blocked.

Configuration (Bahrain example - adjusted per site for local server IPs):

ip access-list extended INTERNET-TO-SERVERS

! Web Server

permit tcp any host 10.0.100.10 eq 80

permit tcp any host 10.0.100.10 eq 443

! Email Server

permit tcp any host 10.0.100.20 eq 25

permit tcp any host 10.0.100.20 eq 110

permit tcp any host 10.0.100.20 eq 143

permit tcp any host 10.0.100.20 eq 993

deny ip any any

Application: Applied inbound on the external (ISP-facing) interface of each site's Internet edge router.
Justification: This configuration hardens the network perimeter by making sure that the only traffic allowed in from the Internet is for public-facing services (Web and Email). All other unsolicited traffic is blocked, protecting the internal network from external threats.

Summary
The project brief says that there needs to be a full inter-department communication policy, and these six ACLs make sure that happens. Only IT staff can manage devices through SSH, ICMP reconnaissance traffic to servers is blocked, users in the same department can work together across sites without any problems, communication between departments is limited based on least privilege, HR and Finance departments can't access the Internet directly, and the attack surface from the Internet is kept to a minimum by only allowing access to certain services on public servers. All ACLs were designed and tested in Cisco Packet Tracer to ensure correct operation without disrupting normal business operations.